INSTRUCTIONS

This form, or an equivalent, may be used to document a rescue plan if not covered in a Work Instruction or similar. Rescue plans must be developed by a competent person, communicated to all people to be involved in implementation, and tested to ensure effectiveness.

| SECTION 1 – Plan Scope |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location of work |  | 2/11 Lenihan ST, Northcote Auckland |  |  |  |  |  | Date |  |  | 11/03/2026 |  |  |  |
| Supervisor name |  | Herman Cronje |  |  |  |  |  | Phone number |  |  | 021446897 |  |  |  |
| Team(s) at risk |  | Planned works |  |  |  |  |  | Number of workers |  |  | 2 |  |  |  |
| Rescue scenario(s) |  | Work at height |  |  |  |  | ☐ | Work in confined space |  |  |  |  |  | ☒ |
|  |  | Isolated or remote work |  |  |  |  | ☐ | Work near water |  |  |  |  |  | ☐ |
|  |  | Live electrical work |  |  |  |  | ☐ | Trench/ tunnel work |  |  |  |  |  | ☐ |
|  |  | Other: |  |  |  |  |  |  |  |  |  |  |  | ☐ |
| SECTION 2 – emergency services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Emergency contact number(s) |  |  |  | 111 |  |  |  | Emergency radio channel |  |  |  |  | VHF Channel 16 / 406 MHz |  |
| Nearest meeting point to meet emergency responders |  |  |  | 12 Lenihan Street, Northcote Point |  |  |  |  |  |  |  |  |  |  |
| SECTION 3 – emergency rescue equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Confirm that all required rescue equipment is on hand and has been checked to confirm it is fit for use: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tripod/ scaffold frame |  |  |  |  |  | Mobile EWP |  |  | Work box and crane |  |  |  |  |  |
| Rescue stretcher |  |  |  |  |  | Rescue/ retrieval kit |  |  | Adjustable lanyards |  |  |  |  |  |
| Retrieval winch |  |  |  |  |  | Drop bar/ T bar |  |  | Belay kit |  |  |  |  |  |
| Poles |  |  |  |  |  | Suspension trauma device |  |  | Ladder |  |  |  |  |  |
| Breathing apparatus |  |  |  |  |  | Davit arm |  |  | Safety harnesses |  |  |  |  |  |
| Radion/ communication kit |  |  |  |  |  | Lone worker alarms/ PLB |  |  | Personal floatation device |  |  |  |  |  |
| LV electrical rescue kit |  |  |  |  |  | HV electrical rescue kit |  |  | Gas detector |  |  |  |  |  |
| Other: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 4 – hazard control plan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identify hazards that must be managed to minimise risk of harm to rescuers and/ or those being rescued. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazard |  |  |  | Control |  |  |  |  |  |  |  |  |  |  |
| Gases |  |  |  | Monitor before, during and after entry with calibrated gas detector |  |  |  |  |  |  |  |  |  |  |
| Slip, trips, falls |  |  |  | Make sure site is always kept clean and tidy |  |  |  |  |  |  |  |  |  |  |
| Manual handling |  |  |  | Use correct lifting technique |  |  |  |  |  |  |  |  |  |  |
| Pedestrians |  |  |  | Close off work zone from any pedestrian access |  |  |  |  |  |  |  |  |  |  |
| Lack of visibility |  |  |  | Where beyond normal visibility, use a head torch |  |  |  |  |  |  |  |  |  |  |
| Poisons |  |  |  | When using glues, primers, concrete, epoxy sika plug or any other solvent, make sure protective gloves and eye protection is always used. |  |  |  |  |  |  |  |  |  |  |
| Battery powered / corded tools |  |  |  | All power tools to be tested and tagged before use |  |  |  |  |  |  |  |  |  |  |
| SECTION 5 – rescue plan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Describe the steps required to safely rescue the person(s) affected, considering the hazard controls identified above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Step | Action |  |  |  |  |  |  |  |  | Person(s) Responsible |  |  |  |  |
| 1 | Identify issue at hand via verbal communication or lack of thereof if issue has escalated. |  |  |  |  |  |  |  |  | Nehal |  |  |  |  |
| 2 | Use retrieval winch to winch the entrant out of the manhole, positioning them to be able exit the narrow opening |  |  |  |  |  |  |  |  | Nehal |  |  |  |  |
| 3 | Once safe, remove clip of winch and identify issue at hand |  |  |  |  |  |  |  |  | Nehal |  |  |  |  |
| 4 | Apply first aid where required once identification of the issue is found |  |  |  |  |  |  |  |  | Nehal |  |  |  |  |
| 5 | Once first aid has been applied, report issue(s) to LH and supervisor |  |  |  |  |  |  |  |  | Nehal |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rescue diagram: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECTION 6 – Rescue Team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| People trained in the use of equipment identified and instructed in the hazard control and rescue plans above. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  |  | Company |  |  |  | Signature |  |  |  |  | Date |  |
| Nehal |  |  |  | Downer |  |  |  |  |  |  |  |  | 11/03/2026 |  |
| Supervisor approval of the plan and confirmation that all required equipment is on hand and rescue team briefed: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |