section 1 – Permit Details

| Contract / Project: |  | Permit number: |  |
| --- | --- | --- | --- |
|  |  | W/O Number: |  |
| Permit issuer: | (Name) (Contact number) |  |  |
| Team leader (permit receiver): | (Name) (Contact number) |  |  |

| Exact location of work: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: ___ / ___ / ___ |  |  |  |  |  |  | From: 7:30_______ am / pm To: ___________ am / pm |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Description of work to be undertaken: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ SWMS, JSEA, attached? AU – only SWMS acceptable |  |  |  |  |  |  |  |  |  |  |
| section 2 – Training & Competency Requirements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Person(s) authorised to enter confined space |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Nominated stand-by person(s) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name |  |  |  |  |  |  | Training expiry |  |  |  |  |  |  |  |  | Name |  |  |  |  |  |  |  |  |  |  | Training expiry |  |  |  |  |  |  |  |
| Minimum training requirements as follows, or equivalent: ☒ Entry team, Stand-by person: AU: RIIOHS202 (or equivalent). NZ: US18426 ☒ Permit receiver also holds: AU: RIIOHS401 (or equivalent), NZ: US17599 ☒ Person(s) monitoring atmosphere: AU: MSMWHS217 Gas test atmospheres. NZ: US25510 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Downer Training Platform |  |  |  |  |  |  |  |
| section 3 – Pre-Entry Safety Checks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Yes |  |  |  | No |  | N/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | No |  |  | N/A |
| - Isolations Verified - Electrical - Hydraulic - Pneumatic - Gas - Water - Stored kinetic energy - Substance supply lines - Other (detail): - Isolation Plan attached |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - PPE and Protective Clothing - Gas detector (calibrated) - Harness / lifeline / tripod (tagged) - Breathing apparatus 1 - ELSA kit (escape set) - Head protection - Hearing protection - Eye / Face protection - Respiratory protection - Safety footwear - Hand protection - Standard overalls - Chemical resistant overalls - Other (detail): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  |  |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  |  | ☐ |  |  | ☒ |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  |  | ☐ |  |  | ☒ |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  |  |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
|  |  |  |  |  | ☐ ☐ |  |  |  | ☐ ☐ |  | ☒ ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
| - Purging / Ventilation - Pre-entry gas check - System cleaned / purged - Natural ventilation sufficient - Air extraction system required - Mechanical ventilation required - Breathing apparatus required 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  |  |
|  |  |  |  |  | ☒ |  |  |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  |  | ☐ |  |  | ☒ |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☒ |  |  |  |  | 1 Planned use of SBA breathing apparatus is not permitted. Signed exemption authority is required. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☐ |  |  |  |  | - Other Precautions - Warning notices / barricades - Adequate lighting - Intrinsically safe equipment only - Hoses / leads protected from harm - Electrical equipment tested / tagged - Smoking restriction communicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  |  |
| - Other Permits (attach all) - Client authority to work permit - Hot-work permit - Other (detail): - Other (detail): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
|  |  |  |  |  | ☒ |  |  |  | ☐ |  | ☐ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☒ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☒ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  | ☐ |
|  |  |  |  |  | ☐ |  |  |  | ☐ |  | ☒ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  | ☐ |  |  |  |
| Additional specific safety instructions: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| section 4 – Emergency Plan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Confined space rescue plan been documented? ☒ Yes, where is it documented?: SWMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Planned means of escape in the event of an emergency |  |  |  | ☐ Self-rescue entry team use ELSA kits to evacuate themselves from the space |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ Assisted rescue (entry team able to be retrieved from outside the confined space): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | No |  |
| Safe means of entry to, and exit from the confined space |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  |  | ☐ |  |
| Effective means of communication between entry team and stand-by person(s) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  |  | ☐ |  |
| First aid facilities and trained personnel readily accessible |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  |  | ☐ |  |
| Effective means of calling for emergency services and/or emergency response team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ |  |  |  | ☐ |  |
| section 5 – Atmospheric Testing * |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Gas detector |  |  | Type: GasAlertMicroClipXL |  |  |  |  |  |  |  |  | Serial number: |  |  |  |  |  |  |  |  |  |  | Calibration due: |  |  |  |  |  |  |  |  |  |  |  |
| Atmospheric monitoring results |  |  | O2 (%) (19.5 to 23.5) |  |  | LEL (%) (max. 5) |  |  |  |  |  |  | H2S (ppm) (max. 10 in AU or 5 in NZ) |  |  |  |  |  | CO (ppm) (max. 30 in AU or 20 in NZ) |  |  |  |  |  | Other: |  |  |  |  | Tester’s signature |  |  |  |  |
| Time of test |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Pre-entry) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (Exit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| section 6 – Work Authorisation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Permit issuer: The risk control measures and precautions appropriate for the safe entry and execution of tasks in the confined space have been implemented and the persons required to enter the confined space have been advised of and understand the requirements of this written authority (Note: This permit is valid only on date of issue). Signed: Date: ____ / ____ / ____ Time: 7:30AM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Permit receiver: I have been advised of and understand the risk control measures and precautions to be observed with the entry and tasks in the confined space. Signed: Date: ____ / ____ / ____ Time: 7:30AM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| section 7 – Revalidation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Suspended (Date/Time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Revalidated (Date/time) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PR Signature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PI Signature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| section 8 – Cancellation of Written Authority |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for permit cancellation: |  | ☐ Work complete (Complete the following checklist, sign below and file permit) |  |  |  |  |  |  |  | ☐ Other: (Sign below to cancel this permit and raise a new permit for remaining work) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work completion checklist |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ | All persons, tools and equipment have exited the confined space and are accounted for |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  |  | Services have been restored (e.g. electricity, water, and gas). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ | Personal locks and tags have been removed and services ready to be restored |  |  |  |  |  |  |  |  |  |  |  |  | ☐ |  |  | Permanent Barriers, signs and covers have been restored. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ | Work area left in safe, clean and tidy state |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Signature of permit receiver |  |  |  |  |  |  |  | Signature of permit issuer |  |  |  |  |  |  |  |  |  |  |  |  | Date: ___ / ___ / ___ Time: |  |  |  |  |  |  |  |  |  |  |  |  |  |

* Attach DG-ZH-FM159 Atmospheric Testing and Sign in Register as required ☐